CEN449 – Project I: Autonomous Navigation Mobile Robot Simulation

Student Name:  Cagri DEMİR

Date: 21 December 2025

Course: CEN449 - Introduction to Autonomous Robots

#### 1. Introduction and Objectives

The primary objective of this project is to simulate an autonomous mobile robot capable of navigating an unknown environment without relying on a pre-saved map. To achieve this, the ROS 2 Humble middleware and the Gazebo simulation environment were utilized.

The specific goals of the project include:

- Setting up a simulation environment with the TurtleBot3 Waffle Pi model.

- Implementing a SLAM (Simultaneous Localization and Mapping) algorithm to generate a 2D occupancy grid map of the environment in real-time.

- Developing a reactive control algorithm to enable the robot to explore the area autonomously while avoiding obstacles using Lidar sensor data.

#### 2. System Architecture

The system is built upon the Robot Operating System (ROS 2 Humble) running on Ubuntu 22.04 LTS. The architecture consists of three main components communicating via ROS 2 topics:

- Simulation Layer: The Gazebo simulator hosts the TurtleBot3 Waffle Pi robot model and the physical environment (turtlebot3_world). It publishes sensor data (LaserScan, Odom, TF).

- Mapping Layer: The Google Cartographer package is used for SLAM. It subscribes to the /scan and /odom topics to construct a real-time map of the environment.

- Control Layer: A custom Python node (otonom_surus.py) serves as the "brain" of the robot. It processes raw Lidar data to detect obstacles and publishes velocity commands to the /cmd_vel topic.

Middleware Configuration: During development, standard ROS 2 discovery issues were encountered. To resolve high latency and packet loss in the simulation, the default middleware was replaced with Eclipse CycloneDDS (rmw_cyclonedds_cpp), ensuring stable communication between nodes.

#### 3. Implementation Details

3.1. Autonomous Navigation Logic Unlike standard navigation stacks that require a known map (e.g., Nav2), this project implements an exploratory navigation approach. A custom Python script was developed to handle obstacle avoidance reactively:

- Sensing: The robot monitors the LaserScan data, specifically focusing on a 20-degree cone in the forward direction.

- Decision Making:

  - If the minimum distance to an obstacle falls below 0.6 meters, the robot halts linear motion and rotates (Angular Velocity: 0.5 rad/s) until the path is clear.

  - If no obstacle is detected, the robot moves forward (Linear Velocity: 0.2 m/s).

3.2. Mapping (SLAM) For mapping, the turtlebot3_cartographer package was launched with the use_sim_time:=True parameter to synchronize with the simulation clock. This allowed the generation of a high-resolution 2D occupancy grid map (.pgm and .yaml) as the robot explored the arena.

#### 4. Results

The simulation was successfully executed, and the robot demonstrated the ability to navigate the entire environment without collision.

Figure 1: Simulation Environment [Figure 1 shows the TurtleBot3 Waffle Pi navigating through the obstacles in the Gazebo environment.

Figure 2: Mapping Output (RViz) [Figure 2 displays the completed occupancy grid map generated by the SLAM algorithm. The black lines represent walls/obstacles, while the white areas represent free space.

#### 5. Conclusion

In this project, a fully autonomous mobile robot simulation was developed using ROS 2. By integrating the Cartographer SLAM algorithm with a custom Python-based obstacle avoidance logic, the robot successfully mapped an unknown environment. The project demonstrated the practical application of ROS 2 nodes, topics, and sensor integration, satisfying all course requirements.